Features

All aluminum structure, fanless design ; Onboard Intel 10th Core i3/i5; Support 1*DDR4 memory slot, Max.Support 32G; Support 1*M.2 2280 slot, 1*2.5”HDD; Support 2*COM (2*COM RS232/485 is Optional); Support DC 12V Input; Support OS：

- Nano N3022                Intel 10th generation core i3 / i5 processor Mini PC

| Specification |  |
| --- | --- |
| Model | Nano N3022 |
| Color | Black (color can be customized) |
| Material | High-quality all aluminum |
| Processor | Intel Core i3 10110U (Dual core 4 threads, 2.1G, Max. Turbo 4.1G, 4MB Cache) |
|  | Intel Core i5 10210U (Quad core 8 threads, 1.6G,Max. Turbo 4.2G, 6MB Cache) |
| CPU Architecture | Comet Lake |
| RAM | Support 1*DDR4 2666/2933 SODIMM memory slot, Max. Support 32GB |
| BIOS | AMI EFI BIOS |
| Display Chip | Intel UHD Graphics for 10th Gen Intel Processors |
| Display Port | 1*DP, 1*HDMI |
| Front I/O | 2*COM (2*COM RS232/485 is Optional), 4*USB3.0, 1*Power Switch, 1*SIM slot |
| Rear I/O | 1*DC-IN 12V, 2*USB2.0, 1*HDMI, 1*DP, 2*Intel 1000M LAN, 1*Audio ( Support Mic and Speaker at the same time) |
| Expansion Slot | Support 1*M.2 2280 slot (Support M.2 PCIE X4 and M.2 SATA SSD); |
|  | Support 1*MINI PCIE slot (WIFI/3G/4G choose one from three),embedded SIM socket; |
| Network | 2*Intel i211-AT 10/100/1000M LAN |
| Storage | Support 1*M.2 2280 slot, 1*2.5”HDD, support RAID 0/1 |
| Power Supply | DC 12V |
| Work Temp | 0℃ ～ +40℃(HDD), -20℃ ～ +50℃(Industrial grade SSD), surface air flow |
| Work Humidity | 5% ～ 95% non-condensing |
| Dimension | 186 * 126.5 * 66 mm |
| Net Weight | 1.3 kg |

| Accessories List |  |  |
| --- | --- | --- |
| Name | Quantity | Specification |
| Power Adapter | 1 | i3 10110U suggest use 12V 5A |
|  |  | i5 10210U suggest use 12V 5A |
| Power Cord | 1 | Any Plug is optional (Chinese, British, USA standard etc) |

| Order Information |  |  |  |
| --- | --- | --- | --- |
| Configuration one | N15 | Intel Core i3 10110U | Standard Port |
| Configuration two |  | Intel Core i5 10210U | Standard Port |

N3022 Drawing

Application:

Business Office, HD Video Playback, Electronic Education, Industrial Automation etc.